# Analysis Questions: Traffic Flow Simulation

## 1. The “Phase Transition” Question

Question:
At exactly what point does traffic spontaneously break down?

In physics, this is similar to asking at what temperature water turns to ice. In traffic systems, we look for the critical density (ρ_c) at which the system transitions from a stable “Free Flow” state to an unstable “Jammed” state.

Parameter to adjust: Density (N)

The experiment:
This analysis corresponds to the Fundamental Diagram (Flow vs Density).

- At low density, traffic flow increases approximately linearly.
- At high density, traffic flow decreases as congestion sets in.

What to look for:
Identify the peak of the flow–density curve. This maximum represents the critical density.

Real-world application:
This value indicates the maximum number of vehicles a highway can support before traffic collapses into gridlock.

## 2. The “Human Factor” Question (Role of Randomness)

Question:
Are traffic jams caused by too many cars, or by driver behaviour?

This experiment examines the role of randomness in traffic flow. The parameter p represents the human factor, including distraction, hesitation, and over-braking.

Parameter to adjust: Random slowing probability (p)

The experiment:
Run the space–time diagram simulation with different values of p.

- p = 0.0: Represents perfectly coordinated driving (e.g. autonomous vehicles).
  Hypothesis: No phantom traffic jams should form; cars move in straight, parallel lines.

- p = 0.5: Represents erratic or distracted driving.
  Hypothesis: Large, unstable traffic jams form quickly, even at moderate densities.

Visual check:
Compare the space–time diagrams. If traffic jams disappear when p = 0, this demonstrates that randomness is essential for the formation of phantom jams.

## 3. The “Speed Limit” Question

Question:
Does lowering the speed limit reduce traffic jams?

Although it seems counter-intuitive, slower speeds can sometimes stabilise traffic flow. Lower speeds reduce braking distances and may prevent stop-and-go behaviour.

Parameter to adjust: Maximum speed (v_max)

The experiment:
Repeat the flow versus density analysis with different speed limits:

- v_max = 5 (default speed limit)
- v_max = 2 (lower speed limit)

What to look for:
Compare the peak (capacity) of the flow–density curves. You may find that at high densities, a lower speed limit maintains a higher and more stable flow.